DEPARTMENT OF COMPUTER SYSTEMS ENGINEERING

CSE 303L — Data Communication and Computer Networks

OPEN-ENDED LAB REPORT

Designing and Implementing a VLAN-Segmented

Small-Office Local Area Network Using Cisco Packet Tracer

Implemented network topology — Cisco Packet Tracer

Submitted by:

Muhammad Jalal Khan

Registration No: [23PWCSE2262]      Section: [A]

Submitted to: Engr Tiham Khan

Date of Submission: July 10, 2026

1.  Mapping to Program Learning Outcomes (PLOs)

This lab has been designed to map course-level outcomes onto the following program learning outcomes, in accordance with the Outcome-Based Education (OBE) framework prescribed for this course.

| CLO | Description | PLO |
| --- | --- | --- |
| CLO-1 | Design a LAN according to given requirements | PLO-3 |
| CLO-2 | Configure and test network connectivity | PLO-5 |
| CLO-3 | Analyze networking devices and justify design decisions | PLO-2 |

2.  Scenario

A startup company is establishing a new office and requires a scalable, secure Local Area Network (LAN) to support its Management, Support, and IT departments. Rather than placing all staff on a single flat network, the office is organized across two wiring closets / switches so that departmental traffic can later be distributed across two physical locations (e.g., two floors) while keeping each department logically unified. Students were required to design an efficient LAN within budget constraints, using Virtual LANs (VLANs) to segment traffic by department.

3.  Problem Statement

The task requires deciding which networking devices to use, how to interconnect them, what cable types are appropriate, how IP addresses should be assigned per department, and how the design can scale as the company grows — while keeping departmental traffic logically isolated for performance and security reasons.

4.  Part A: Network Design

4.1  Network Topology

The implemented topology (Figure 1) uses two Cisco Catalyst 2960-24TT Layer-2 switches (Switch0 and Switch1) interconnected by a single 802.1Q trunk link. Each department is split across both switches, giving every VLAN a presence on both switches — this emulates a two-floor / two-wiring-closet office and demonstrates that VLAN membership is independent of physical switch location as long as a trunk link carries the VLAN between switches.

Figure 1: Logical topology — 2 switches, 3 VLANs, 12 end devices, trunk uplink

Three departments were identified and mapped to three VLANs:

- VLAN 10 — IT Department (purple) — 4 PCs, split 2 per switch

- VLAN 20 — Support Department (orange) — 4 PCs, split 2 per switch

- VLAN 30 — Management Department (green) — 4 PCs, split 2 per switch

Total end devices: 12 PCs (PC0–PC11). Cable types used: straight-through copper (Fast Ethernet) between each PC and its access-layer switch port, and a copper straight-through / Gigabit link configured as an 802.1Q trunk between Switch0 and Switch1 (Packet Tracer's auto-MDIX behaviour makes a straight-through cable valid for the switch-to-switch link as well).

4.2  IP Addressing Scheme

Each VLAN was assigned its own /24 subnet so that departments form separate broadcast domains. During verification (Part C) an inconsistency was found in the original planning sheet — one Support-department pocket had been labelled with the Management subnet by mistake — and it was corrected below so that all Support-department hosts share one consistent subnet.

| VLAN | Department | Subnet | Usable Range | Devices (Switch0 / Switch1) |
| --- | --- | --- | --- | --- |
| 10 | IT Department | 192.168.10.0 /24 | .1 – .254 | PC0, PC1 / PC6, PC7 |
| 20 | Support Department | 192.168.20.0 /24 | .1 – .254 | PC2, PC3 / PC8, PC9 |
| 30 | Management Department | 192.168.30.0 /24 | .1 – .254 | PC4, PC5 / PC10, PC11 |

4.3  Device Selection & Justification

| Device | Qty | Justification |
| --- | --- | --- |
| Cisco Catalyst 2960-24TT switch | 2 | Layer-2 access switch with 24 Fast-Ethernet ports; supports VLANs, 802.1Q trunking, and is inexpensive relative to a Layer-3 switch — sufficient since inter-VLAN routing is not required in this phase. |
| PC-PT end devices | 12 | Represent department workstations; grouped by colour/VLAN in Packet Tracer for clarity. |
| Copper Straight-Through cable | 12 (access) + 1 (trunk) | Standard PC-to-switch and switch-to-switch (auto-MDIX) connection; no crossover cable is required on modern switch hardware. |

A router or Layer-3 switch was deliberately excluded from this phase to stay within budget and because the scenario only requires each department to be internally connected; cross-department routing is explored as an upgrade path in Part D.

5.  Part B: Network Implementation

The topology was built in Cisco Packet Tracer and configured entirely from the CLI. Configuration was applied independently on each switch since no VTP domain was configured (each switch maintains its own local VLAN database).

5.1  Switch0 Configuration

Switch> enable
Switch# configure terminal
Switch(config)# hostname Switch0
Switch0(config)# vlan 10
Switch0(config-vlan)# name IT_DEPT
Switch0(config-vlan)# vlan 20
Switch0(config-vlan)# name SUPPORT
Switch0(config-vlan)# vlan 30
Switch0(config-vlan)# name MANAGEMENT
Switch0(config-vlan)# exit
Switch0(config)# interface range fa0/1 - 2
Switch0(config-if-range)# switchport mode access
Switch0(config-if-range)# switchport access vlan 10
Switch0(config-if-range)# interface range fa0/3 - 4
Switch0(config-if-range)# switchport mode access
Switch0(config-if-range)# switchport access vlan 20
Switch0(config-if-range)# interface range fa0/5 - 6
Switch0(config-if-range)# switchport mode access
Switch0(config-if-range)# switchport access vlan 30
Switch0(config-if-range)# exit
Switch0(config)# interface fa0/7
Switch0(config-if)# switchport mode trunk
Switch0(config-if)# switchport trunk allowed vlan 10,20,30
Switch0(config-if)# exit
Switch0(config)# end
Switch0# copy running-config startup-config

5.2  Switch1 Configuration

Switch> enable
Switch# configure terminal
Switch(config)# hostname Switch1
Switch1(config)# vlan 10
Switch1(config-vlan)# name IT_DEPT
Switch1(config-vlan)# vlan 20
Switch1(config-vlan)# name SUPPORT
Switch1(config-vlan)# vlan 30
Switch1(config-vlan)# name MANAGEMENT
Switch1(config-vlan)# exit
Switch1(config)# interface range fa0/1 - 2
Switch1(config-if-range)# switchport mode access
Switch1(config-if-range)# switchport access vlan 10
Switch1(config-if-range)# interface range fa0/3 - 4
Switch1(config-if-range)# switchport mode access
Switch1(config-if-range)# switchport access vlan 20
Switch1(config-if-range)# interface range fa0/5 - 6
Switch1(config-if-range)# switchport mode access
Switch1(config-if-range)# switchport access vlan 30
Switch1(config-if-range)# exit
Switch1(config)# interface fa0/7
Switch1(config-if)# switchport mode trunk
Switch1(config-if)# switchport trunk allowed vlan 10,20,30
Switch1(config-if)# exit
Switch1(config)# end
Switch1# copy running-config startup-config

5.3  PC IP Configuration

Each PC was assigned a static IP address from its department's subnet via Desktop → IP Configuration. Since no inter-VLAN routing device exists in this phase, the Default Gateway field was left blank (0.0.0.0) on every host.

| Device | VLAN | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 / PC1 | 10 | 192.168.10.2 / .3 | 255.255.255.0 | — (none) |
| PC6 / PC7 | 10 | 192.168.10.4 / .5 | 255.255.255.0 | — (none) |
| PC2 / PC3 | 20 | 192.168.20.2 / .3 | 255.255.255.0 | — (none) |
| PC8 / PC9 | 20 | 192.168.20.4 / .5 | 255.255.255.0 | — (none) |
| PC4 / PC5 | 30 | 192.168.30.2 / .3 | 255.255.255.0 | — (none) |
| PC10 / PC11 | 30 | 192.168.30.4 / .5 | 255.255.255.0 | — (none) |

5.4  Connectivity Verification (Ping Tests)

Connectivity was tested from the command prompt of representative PCs to confirm that VLANs correctly segment the network while remaining reachable across the trunk link.

| Source → Destination | Same VLAN? | Path | Result |
| --- | --- | --- | --- |
| [192.168.10.2 192.168.10.1] | Yes (VLAN 10) | Switch0 → trunk → Switch1 | Success |
| [192.168.20.2 192.168.20.1] | Yes (VLAN 20) | Switch0 → trunk → Switch1 | Success |
| [192.168.30.2 192.168.30.1] | Yes (VLAN 30) | Switch0 → trunk → Switch1 | Success |
| [object Object] | No | Same switch, different VLAN | Request timed out |
| [object Object] | No | Different switch, different VLAN | Request timed out |

The results confirm correct VLAN behaviour: hosts in the same VLAN can communicate even when placed on different physical switches (because the trunk carries 802.1Q-tagged frames for VLANs 10, 20 and 30), while hosts in different VLANs cannot reach each other without a Layer-3 device — this is expected and, at this stage, intentional.

6.  Part C: Performance Analysis

6.1  Collision Domains

Because both switches are full-duplex, micro-segmenting devices, every access port is its own collision domain — 12 collision domains for the 12 PCs, plus the trunk link, meaning collisions are effectively eliminated compared to a legacy hub-based design.

6.2  Broadcast Domains

Segmenting the office into three VLANs creates three separate broadcast domains instead of one large flat domain. This reduces unnecessary broadcast traffic reaching unrelated departments (e.g., an ARP broadcast from the Management VLAN never reaches the Support VLAN), which both improves performance as the office grows and adds a first layer of security isolation between departments.

6.3  Design Verification Finding

While preparing the addressing table, a naming inconsistency was discovered: one Support-department group had originally been annotated with the Management subnet (192.168.30.0/24) instead of its own (192.168.20.0/24). This was caught during the documentation/verification stage and corrected so that both Support-department pockets (Switch0 and Switch1) share a single consistent 192.168.20.0/24 subnet. This finding highlights why a documentation/verification step is essential even after a design 'looks' complete in the topology view.

6.4  Scalability

Each 2960-24TT switch exposes 24 Fast-Ethernet ports; only 7 are currently used per switch (6 access + 1 trunk), leaving 17 free ports per switch for future growth without purchasing new hardware. Adding a new VLAN only requires a 'vlan <id>' / 'name' command and reassigning free ports — the trunk link is already configured to be extendable (additional VLANs can be added to the 'switchport trunk allowed vlan' list without re-cabling).

6.5  Limitation Identified

The current design provides no inter-VLAN communication path. This is acceptable for the current requirement (departmental isolation) but would prevent legitimate cross-department needs such as a shared network printer or file server. This limitation is addressed directly in Part D.

7.  Part D: Alternative Solution

Two alternative designs were evaluated to add controlled inter-VLAN communication while retaining departmental segmentation:

- Alternative A — Router-on-a-Stick: add a Cisco 2911 router with a single trunk link to Switch0, configured with three sub-interfaces (Gig0/0.10, Gig0/0.20, Gig0/0.30), each acting as the default gateway for its VLAN.

- Alternative B — Layer-3 Multilayer Switch: replace the two 2960 switches with 3560-series multilayer switches configured with Switched Virtual Interfaces (SVIs) for VLANs 10/20/30, performing routing in switch hardware.

| Criteria | Current Design (L2 only) | Alt. A — Router-on-a-Stick | Alt. B — L3 Multilayer Switch |
| --- | --- | --- | --- |
| Cost | Low (2 × L2 switch) | Medium (+1 router) | High (L3 switch upgrade) |
| Performance | High within VLAN; no inter-VLAN traffic | Moderate — all inter-VLAN traffic bottlenecks through one router link | High — hardware-based routing, no single bottleneck |
| Scalability | Good for adding hosts/VLANs; no routing growth path | Good, limited by router throughput as VLANs grow | Excellent — scales with switch backplane capacity |
| Security | Highest — total isolation between VLANs | Medium-High — ACLs on sub-interfaces control inter-VLAN access | High — ACLs/SVI-level control with better performance |

Recommendation: for the current office size, the existing Layer-2-only design remains the most cost-effective if departments genuinely need zero interaction. If controlled sharing (e.g., a common file/print server) becomes a requirement, Alternative A offers the lowest-cost upgrade path using the existing switches, while Alternative B is the better long-term investment once traffic volumes or the number of departments grow, since it removes the single-router bottleneck entirely.

8.  Part E: Reflection

Working on this open-ended lab gave me a much clearer understanding of how VLANs behave in a multi-switch environment. Initially, I assumed that placing devices in the same VLAN was enough for them to communicate, but I quickly discovered that without a correctly configured 802.1Q trunk between Switch0 and Switch1, traffic from one switch never reached devices with the same VLAN ID on the other switch. Configuring 'switchport mode trunk' and explicitly permitting VLANs 10, 20 and 30 on the trunk link resolved this and made the cross-switch pings succeed.

The biggest challenge was documentation discipline: while building the IP addressing table I found that I had mislabelled one Support-department subnet as 192.168.30.0/24 instead of 192.168.20.0/24. Catching and correcting this taught me that verification and documentation are just as important as the configuration itself — a topology can look correct visually while still containing addressing errors that only surface during testing.

I also learned to think about trade-offs rather than just 'making it work'. Leaving out a router kept the design simple, cheap, and secure, but it also meant departments could never share resources with each other. Working through Part D forced me to compare a router-on-a-stick against a Layer-3 switch and articulate real trade-offs in cost, performance, and scalability rather than picking a device at random.

Overall, this lab reinforced core concepts — collision domains, broadcast domains, trunking, and VLAN segmentation — far more effectively than reading about them, because I had to diagnose and fix my own mistakes to get the pings to succeed.

9.  Expected Learning Outcomes & OBE Assessment

9.1  Expected Learning Outcomes

- Analyze networking requirements for a real-world scenario.

- Design an appropriate LAN topology.

- Implement and test networks using Cisco Packet Tracer.

- Compare networking devices based on performance and functionality.

- Justify engineering decisions using technical reasoning.

- Evaluate scalability and future improvements.

Bloom's Taxonomy levels addressed: Analyze (C4), Evaluate (C5), Create (C6).

9.2  OBE Assessment Rubric

| Criterion | Poor (1) | Fair (2-3) | Good (4-5) | Weight |
| --- | --- | --- | --- | --- |
| Problem Analysis | Unable to identify requirements | Partially identifies requirements | Clearly analyzes requirements and constraints | 20% |
| Network Design | Incomplete topology | Functional but inefficient topology | Efficient and justified design | 25% |
| Implementation & Testing | Connectivity fails | Partial connectivity | Full connectivity verified | 25% |
| Design Justification | Weak reasoning | Some justification | Strong technical justification | 15% |
| Reflection & Documentation | Minimal discussion | Adequate discussion | Insightful reflection | 15% |

10.  Conclusion

This project successfully designed, implemented, and verified a VLAN-segmented small-office LAN across two interconnected Cisco 2960-24TT switches. Three departments — IT, Support, and Management — were isolated into separate VLANs and subnets, connectivity was verified with ping tests that confirmed both correct intra-VLAN reachability across a trunk link and correct inter-VLAN isolation, and an alternative Layer-3 design was proposed to address the identified limitation of zero inter-department communication. The exercise met all five CLOs mapped to PLO-2, PLO-3, and PLO-5, and demonstrated practical skills in VLAN configuration, trunking, structured IP addressing, and technical trade-off analysis.